Dimensionality Reduction and Multicollinearity Elimination of AGV Sensor Data Based on Correlation Analysis and PCA

Nikita Morkva, Anastasiia Zabolotna, Ivan Hrytsev

Lviv Polytechnic National University, Institute of Computer Science and Information Technologies (ICSIT)

Lviv, Ukraine

Email: { nikita.morkva.oi.2025, anastasiia.zabolotna.oi.2024, ivan.hrytsev.oi.2025 }@lpnu.ua

Abstract—This paper presents a systematic study of dimensionality reduction and multicollinearity handling for AGV (Automated Guided Vehicle) battery sensor data in the context of LSTM-based voltage forecasting. Six input representations are compared: raw sensor data (Baseline), physics-informed feature engineering (Stage 14 with ΔV, dV/dt, moving average, and SOC), Linear PCA (88 % explained variance), Kernel PCA (RBF), Random Forest Top-5 feature selection, and Factor Analysis, evaluated across three sliding-window sizes (40, 60, and 80 time steps). Results show that feature engineering with window = 60 achieves the best accuracy (R² = 0.9967), Linear PCA provides an acceptable 2.5× compression trade-off, and Factor Analysis fundamentally fails due to its treatment of the monotone discharge trend as noise. We further demonstrate that multicollinearity among telemetry features does not impair LSTM performance; correlated features provide complementary temporal context that improves forecast stability.

Keywords—AGV, battery voltage forecasting, PCA, LSTM, feature engineering, multicollinearity, dimensionality reduction, time series

# I. Introduction

Automated Guided Vehicles (AGVs) generate dense multivariate sensor streams: battery voltage V(t), discharge current I(t), State of Charge (SOC), kinematic parameters such as wheel speed, and wireless signal metrics (RSSI). Accurate forecasting of the battery voltage trajectory is fundamental to safe operation, route planning, and predictive maintenance in autonomous logistics. The feature space, however, is characterised by strong inter-feature correlations, raising a classical concern: does multicollinearity degrade model quality, and should dimensionality reduction be applied before training recurrent neural networks?

Existing literature addresses these questions separately. Correlation-based feature selection [1], PCA-based preprocessing [2], and LSTM architectures for battery state estimation [3] have each been studied in isolation. A consolidated study that systematically compares multiple reduction strategies on the same AGV dataset, with controlled window sizes and identical LSTM architectures, is, however, absent. This paper fills that gap.

Our contributions are: (1) a rigorous comparative benchmark of six preprocessing methods under identical LSTM conditions; (2) an analysis of the interaction between sliding-window size and input representation quality; and (3) practical guidance on when and why dimensionality reduction helps or hurts for electrochemical time-series forecasting.

# II. Methodology

## A. Dataset and Target Variable

The dataset consists of time-stamped AGV battery telemetry resampled to a 10-second interval. The prediction target is next-step battery cell voltage V(t + 1), expressed in millivolts (mV). Data is partitioned chronologically into training, validation, and test sets to prevent information leakage. Outliers are removed using the Modified Z-score (threshold = 6) and smoothed with a Savitzky–Golay filter (window = 71, polynomial order = 2).

## B. Feature Engineering — Stage 14

The raw feature vector is augmented with four physics-informed signals that expose the model to the dynamics of the discharge process rather than its instantaneous state alone:

- ΔV = V(t) − V(t−1) — discrete first-order voltage change;

- dV/dt — continuous rate of voltage fall, directly proportional to internal resistance growth during degradation;

- Speed_MA — moving average of wheel speed (window = 5) for noise suppression;

- SOC(t) — State of Charge via cumulative discharge, normalised to the observed maximum.

The resulting 16-dimensional vector provides explicit degradation-velocity context. Adding these derivative features produced the largest accuracy gain observed in the study, confirming that domain knowledge about discharge physics is more valuable than unsupervised compression.

## C. Correlation and Multicollinearity Analysis

Three complementary statistics are computed. Pearson’s r measures linear dependence; Spearman’s ρ and Kendall’s τ capture monotone non-linear relations. Heat-map inspection identifies three high-correlation clusters: (i) voltage–current–SOC (r > 0.95); (ii) wheel speed–energy consumption (r ≈ 0.92); (iii) GPS coordinates–RSSI (r ≈ 0.87). Variance Inflation Factor (VIF) values exceed 10 for several features, confirming textbook multicollinearity. A key finding of this study, however, is that such multicollinearity does not impair the LSTM: correlated features carry temporally shifted versions of the same signal, providing the gated memory cells with richer temporal context.

## D. Dimensionality Reduction Strategies

Linear PCA decomposes the feature covariance matrix into eigenvectors sorted by explained variance. Retaining components that account for 88 % of total variance compresses the 16-dimensional space to 6 principal components—a 2.5× reduction—while eliminating correlated directions by construction. Kernel PCA with an RBF kernel maps features into a reproducing kernel Hilbert space before eigendecomposition, theoretically capturing non-linear structure. Factor Analysis (FA) with Varimax rotation decomposes the correlation matrix into latent common factors, but assumes stationary covariance structure and explicitly treats monotone trends as noise (communality is attributed to shared factors, while the trend is placed in the unique variance component). Random Forest importance ranking identifies the five features with the highest aggregate impurity reduction, providing a data-driven sparse selection.

## E. LSTM Architecture and Sliding-Window Setup

A sliding-window transform converts the multivariate series into supervised samples: X_t = [X(t−w + 1), …, X(t)], y = V(t + 1). Three window sizes w ∈ {40, 60, 80} are evaluated. The architecture is: Input(w × d) → LSTM(64, 64 units) → Dropout(0.2) → Dense(1). Training uses Adam, batch = 16, up to 100 epochs with early stopping (patience = 10) and L1 regularisation (λ = 1e–4). All models are compared on the same held-out chronological test split.

# III. Experimental Results

## A. Performance at Window = 40 (Under-Context Regime)

At the shortest window of 40 time steps (≈ 7 minutes of telemetry), Raw Baseline achieves the highest R² = 0.9586. Notably, the Enhanced Raw Data (Stage 14) scores only R² = 0.8834 despite its richer feature set. The explanation is that derivative features such as dV/dt and ΔV need sufficient historical context to stabilise: with only 40 steps, the LSTM cannot fully exploit the temporal structure embedded in the derivatives, and the extra dimensions add noise relative to the information gain. Linear PCA collapses to R² = 0.5064 because the compressed 6-component representation lacks the temporal resolution needed to reconstruct the discharge curve at this short horizon. Factor Analysis produces R² = −60.60, confirming a catastrophic mismatch.

TABLE I. Comparative Performance (Window = 40)

| Method (Input Data) | R² | MAE (mV) | RMSE (mV) | MAPE (%) |
| --- | --- | --- | --- | --- |
| Raw Data (Baseline) | 0.9586 | 96.43 | 111.26 | 0.2020 |
| Random Forest Top-5 | 0.9297 | 136.81 | 145.06 | 0.2859 |
| Enhanced Raw Data (Stage 14) | 0.8834 | 143.62 | 186.75 | 0.2986 |
| Kernel PCA | 0.6673 | 259.49 | 315.50 | 0.5417 |
| Linear PCA | 0.5064 | 328.66 | 384.29 | 0.6864 |
| Factor Analysis | −60.60 | 4269.59 | 4293.09 | 8.9436 |

All error metrics in mV.

## B. Performance at Window = 60 (Optimal Temporal Horizon)

At window = 60 (≈ 10 minutes), a striking reversal occurs. The Enhanced Raw Data climbs to R² = 0.9967—the absolute peak across all conditions. This is the synergy “sweet spot”: with 60 steps of context, the LSTM can resolve the physics-informed derivatives, the moving average has fully converged, and SOC provides meaningful long-range drift information. The Raw Baseline falls to R² = 0.9142, demonstrating that without the engineered features, a longer window yields diminishing returns. Linear PCA recovers strongly to R² = 0.9069, confirming that the compressed representation requires at least 60 steps to convey enough trend information. Kernel PCA remains at R² = 0.5183, further evidence that artificial non-linearity hurts a quasi-linear discharge process.

TABLE II. Comparative Performance (Window = 60)

| Method (Input Data) | R² | MAE (mV) | RMSE (mV) | MAPE (%) |
| --- | --- | --- | --- | --- |
| Enhanced Raw Data (Stage 14) | 0.9967 | 22.28 | 28.28 | 0.0469 |
| Random Forest Top-5 | 0.9812 | 53.18 | 67.64 | 0.1113 |
| Raw Data (Baseline) | 0.9142 | 131.83 | 144.44 | 0.2762 |
| Linear PCA | 0.9069 | 113.24 | 150.42 | 0.2383 |
| Kernel PCA | 0.5183 | 251.17 | 342.18 | 0.5281 |
| Factor Analysis | −56.22 | 3717.25 | 3729.57 | 7.8088 |

All error metrics in mV.

## C. Performance at Window = 80 (Overfitting Regime)

Extending the window to 80 steps (≈ 13 minutes) degrades most methods relative to window = 60. The Raw Baseline recovers to R² = 0.9858, but Enhanced Raw Data drops to R² = 0.9508. The degradation occurs because a very long window causes the LSTM to overfit on micro-fluctuations and short-term battery recovery artefacts—brief voltage rebounds caused by rest phases between duty cycles—that are irrelevant to the macro discharge trend. These artefacts are amplified by the derivative features (ΔV, dV/dt), which are sensitive to short-term oscillations. Kernel PCA improves notably to R² = 0.8464 at this window, suggesting that its non-linear kernel can partially exploit the additional history, but it still lags behind simpler methods. Factor Analysis worsens further to R² = −68.12.

TABLE III. Comparative Performance (Window = 80)

| Method (Input Data) | R² | MAE (mV) | RMSE (mV) | MAPE (%) |
| --- | --- | --- | --- | --- |
| Raw Data (Baseline) | 0.9858 | 58.63 | 61.90 | 0.1234 |
| Random Forest Top-5 | 0.9734 | 71.31 | 84.82 | 0.1495 |
| Enhanced Raw Data (Stage 14) | 0.9508 | 105.93 | 115.31 | 0.2228 |
| Kernel PCA | 0.8464 | 146.99 | 203.80 | 0.3068 |
| Linear PCA | 0.7166 | 206.99 | 276.88 | 0.4360 |
| Factor Analysis | −68.12 | 4302.56 | 4323.87 | 9.0232 |

All error metrics in mV.

# IV. Discussion

## A. Multicollinearity Is Not a Problem for LSTM

A central result of this study is that multicollinearity among sensor features does not degrade, and in fact benefits, LSTM-based voltage forecasting. Classical regression theory prescribes removing correlated predictors because multicollinearity inflates coefficient variance. LSTMs, however, are non-parametric and exploit temporal co-variation: correlated features provide the gated memory cells with shifted and scaled versions of the same underlying physical signal, effectively giving the model multiple “views” of the same electrochemical event. Random Forest importance scores confirm this empirically—every feature in the 16-dimensional set, including those with VIF > 10, receives non-zero importance weight, indicating that none are truly redundant in the LSTM sense.

## B. PCA as the Best Engineering Trade-off

Linear PCA at 88 % explained variance achieves a 2.5× dimensionality compression—from 16 to 6 features—with only a modest accuracy penalty. At the optimal window of 60, Linear PCA reaches R² = 0.9069 versus R² = 0.9142 for the Raw Baseline: a difference of 0.73 percentage points in R² for a 62.5 % reduction in feature count. For resource-constrained embedded AGV controllers where inference latency and model memory are limited, this is an attractive trade-off. Importantly, Linear PCA also resolves multicollinearity by construction, which simplifies downstream model interpretation.

## C. Factor Analysis: Fundamental Mismatch

Factor Analysis fails catastrophically across all conditions (R² ranging from −56 to −68). The root cause is a fundamental methodological mismatch. FA models the covariance matrix as a sum of shared latent factors and unique variances, assuming that observations are drawn from a stationary distribution. Battery discharge, however, is a non-stationary monotone process: the voltage decreases quasi-linearly from approximately 48 000 mV to 47 000 mV across the measurement window. FA interprets this dominant trend as a unique variance component (noise) rather than as a shared signal, effectively discarding the most informative feature of the data before the LSTM ever sees it. This finding generalises: FA should never be used as a preprocessing step for time-series with strong deterministic trends.

## D. Window Size and Method Synergy

The window-size analysis reveals a clear three-regime pattern. In the under-context regime (w = 40), raw features dominate because derivative features have not yet converged. In the optimal regime (w = 60), feature engineering reaches peak performance as the LSTM fully exploits physics-informed temporal context. In the overfitting regime (w = 80), micro-fluctuations in the derivative features cause the model to overfit on irrelevant battery recovery artefacts, degrading the engineered representation relative to raw features. The implication for deployment is that window size and feature representation are co-dependent hyperparameters and must be tuned jointly.

# V. Conclusions

This paper delivered five empirically grounded conclusions:

- Feature engineering with physics-informed derivatives (dV/dt, ΔV) and a moving average smoother achieves the highest accuracy (R² = 0.9967) at window = 60 and is recommended when forecast accuracy is the primary objective.

- Linear PCA (88 % variance, 2.5× compression) provides the best engineering trade-off: R² = 0.9069 at window = 60, suitable for embedded deployment with strict resource constraints.

- Multicollinearity among AGV sensor features is not a problem for LSTM networks; correlated features supply complementary temporal context and should be retained when accuracy matters.

- Kernel PCA and Factor Analysis are unsuitable for quasi-linear battery discharge forecasting. Kernel PCA introduces artificial non-linearity; Factor Analysis discards the monotone trend as noise.

- Window size and feature representation are co-dependent: the optimal window is w = 60 for feature-engineered inputs, while raw features perform better at the extremes (w = 40 and w = 80).

Future work will investigate multi-step probabilistic forecasting, attention-augmented LSTM variants, and transfer of the trained pipeline across AGV fleets with heterogeneous battery chemistries.

# Acknowledgment

The authors thank the Institute of Computer Science and Information Technologies of Lviv Polytechnic National University for access to AGV telemetry infrastructure and computational resources used in this research.

# References

[1] I. Goodfellow, Y. Bengio, and A. Courville, Deep Learning. Cambridge, MA: MIT Press, 2016.

[2] S. Hochreiter and J. Schmidhuber, “Long short-term memory,” Neural Computation, vol. 9, no. 8, pp. 1735–1780, 1997.

[3] I. T. Jolliffe, Principal Component Analysis, 2nd ed. New York: Springer, 2002.

[4] L. Breiman, “Random forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, 2001.

[5] R. Tibshirani, “Regression shrinkage and selection via the lasso,” J. Roy. Stat. Soc. B, vol. 58, no. 1, pp. 267–288, 1996.

[6] B. Schölkopf, A. Smola, and K.-R. Müller, “Kernel principal component analysis,” in Proc. ICANN, 1997, pp. 583–588.

[7] Z. Chen, M. Wu, R. Zhao, F. Guretno, and X. Yan, “Machine remaining useful life prediction via an attention-based deep learning approach,” IEEE Trans. Ind. Electron., vol. 68, no. 3, pp. 2521–2531, 2021.